Copyright 2008 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 08 12 16 - ALUMINUM DOOR FRAMES

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes <Insert description of essential item included in the Section>.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Division <Insert MF04 Division number> Section "<Insert MF04 Section title>" for <Insert description of related work>.

      - DEFINITIONS (RESERVED)

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference:  Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Division 01 Section "Project Management and Coordination."

      - ACTION SUBMITTALS

Do not ask for manufacturer's specifications, material safety data sheets, installation instructions, and similar items that may conflict with specification requirements or needlessly involve Architect in Contractor's means and methods of construction.

        - Product Data:  For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for <Insert product>.

          - Include rated capacities, operating characteristics, [electrical characteristics,] and furnished specialties and accessories.

Include Shop Drawings only for work that involves custom fabrication or if Shop Drawings are typically prepared to assist in the performance of the work involved.

        - Shop Drawings:

          - Include plans, elevations, sections, and [mounting] [attachment] details.

          - Include details of equipment assemblies.  Indicate dimensions, weights, loads, required clearances, method of field assembly, components, and location and size of each field connection.

          - Detail fabrication and assembly of <Insert portion of Work>.

Retain subparagraph below if equipment includes wiring.

          - Include diagrams for power, signal, and control wiring.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable.  Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

        - Samples:  For each exposed product and for each color and texture specified, <Insert dimensions> in size.

        - Samples for Initial Selection:  For [each type of unit.] [each type of exposed finish] [units with factory-applied finishes].

        - Samples for Verification:  For [each type of unit.] [each type of exposed finish.] [the following products, in manufacturer's standard sizes:]

          - <Insert list of products>.

        - Product Schedule:  For <Insert product>.[  Use same designations indicated on Drawings.]

Insert other kinds of action submittals such as schedules (for subjects such as planting and hardware), quality-control submittals, and restoration programs as necessary for the Section.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

        - Delegated-Design Submittal:  For <Insert product or assembly>.

      - INFORMATIONAL SUBMITTALS

Retain "Coordination Drawings" Paragraph below for situations where limited space necessitates maximum utilization for efficient installation of different components or if coordination is required for installation of products and materials by separate installers.  Coordinate paragraph with other Sections specifying products listed below.  Preparation of coordination drawings requires the participation of each trade involved in installations within the limited space.

        - Coordination Drawings:  <Insert description of drawing type> [and other details], drawn to scale, on which the following items are shown and coordinated with each other, using input from installers of the items involved:

          - <Insert requirements>.

Coordinate "Qualification Data" Paragraph below with qualification requirements in Division 01 Section "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data:  For [Installer] [Applicator] [manufacturer] [fabricator] [testing agency] [factory-authorized service representative] <Insert entity or specialist>.

Retain "[Material] [Product] Certificates" Paragraph below to require submittal of certificates from manufacturers.

        - [Material] [Product] Certificates:  For each type of <Insert material or product>.

          - <Insert list of materials or products>.

Retain "Material Test Reports" Paragraph below for material test reports that are Contractor's responsibility.

        - Material Test Reports:  For each <Insert material>, by a qualified testing agency.

Requirements in first paragraph below are for test reports for products on which tests are performed either by independent testing agencies or by manufacturers in their own labs.  Retain first option if testing is likely to be performed in manufacturer's facilities and witnessed by a qualified testing agency; retain second option if testing is performed by the testing agency.

        - Product Test Reports:  For each <Insert product>, for tests performed by [manufacturer and witnessed by a qualified testing agency] [a qualified testing agency].

        - Evaluation Reports:  For <Insert products, assemblies, or systems>, from ICC-ES.

        - Source quality-control reports.

Retain "Field quality-control reports" Paragraph below if Contractor is responsible for field quality-control testing and inspecting.

        - Field quality-control reports.

        - Sample Warranty:  For [manufacturer's] [special] warranty.

Insert other kinds of informational submittals such as design data and manufacturer's field reports as necessary for the Section.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data:  For <Insert products or materials> to include in maintenance manuals.

      - MAINTENANCE MATERIAL SUBMITTALS

        - Furnish extra materials[, from the same product run,] that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - <Insert product>:  [Full-size units] <Insert detailed description of material and form making up a unit> equal to <Insert number> percent of quantity installed [for each size indicated], but no fewer than <Insert number> units.

      - QUALITY ASSURANCE

Include in this article prerequisites, standards, limitations, and criteria that establish an overall level of quality for products and workmanship.  Include only those quality-assurance provisions that affect all Work of this Section rather than those that apply only to Part 2 - Products or Part 3 - Execution.

        - Manufacturer Qualifications:  <Insert Section-related qualifications>.

        - Fabricator Qualifications:  <Insert Section-related qualifications>.

Retain one of two "Installer Qualifications" paragraphs below.

        - Installer Qualifications:  Fabricator of products.

        - Installer Qualifications:  [An entity that employs installers and supervisors who are trained and approved by manufacturer] [An authorized representative who is trained and approved by manufacturer].

      - DELIVERY, STORAGE, AND HANDLING (RESERVED)

Retain this article only if providing detailed requirements that apply to Work of this Section and that supplement general provisions inDivision 01 Section "Product Requirements."

      - FIELD CONDITIONS (RESERVED)

Retain this article only if providing detailed requirements that apply to Work of this Section and that supplement general provisions in Division 01.

Include requirements for field measurements by Contractor that apply specifically to Work of this Section.  Do not include temporary procedure requirements such as protecting the site; include these requirements in "Preparation" Article.

      - WARRANTY

Include special Project warranties only in those Sections where availability has been verified with contractors and manufacturers listed.

If Installer is included as signer of the warranty, the maximum warranty period that sureties will allow is two years.  Use specified form of special warranty only when it is included at the end of the Section.

When warranties are required, verify with Owner's counsel that warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - [Manufacturer's] [Special] Warranty:  [Manufacturer agrees] [Installer agrees] [Manufacturer and Installer agree] to repair or replace [components of] <Insert product> that fail(s) in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods.

          - Warranty Period:  <Insert number> year(s) from date of Substantial Completion.

- PRODUCTS

For an explanation of options and Contractor’s product selection procedures, see Division 01 Section "Product Requirements."

      - MANUFACTURERS

        - Basis-of-Design Product:  Subject to compliance with requirements, provide Classic Frame System, by RACO Interior Products, Inc., or comparable product by one of the following:

          - Western Integrated Materials, Inc., www.western-integrated.com.

      - MATERIALS (RESERVED)

      - COMPONENTS (RESERVED)

Titles in this article can either stand alone or be combined with "General" articles as described previously, depending on the nature of the Section's contents and for shorter, less complex articles.  Avoid repeating common requirements.

      - ACCESSORIES (RESERVED)

      - GENERAL FINISH REQUIREMENTS

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

Retain paragraph above only in Sections where manufacturers typically provide strippable, temporary protective coverings for their products.  Retain first paragraph below only in Sections where finishing typically occurs after assembly; qualify if only applicable to some finishes in the Section.

        - Finish products after assembly.

- EXECUTION

      - EXAMINATION

Retain this article only if it supplements Division 01 requirements and includes provisions that apply specifically to individual Sections.

        - Examine [substrates] [areas] [and] [conditions][, with Installer present,] [, with Applicator present,] for compliance with requirements for [maximum moisture content,] installation tolerances [and] <Insert other specific conditions> and other conditions affecting performance of the Work.

First paragraph below is for products and materials that are subject to mold.

        - Examine <Insert products or materials> before installation.  Reject <Insert products or materials> that are wet, moisture damaged, or mold damaged.

        - Examine roughing-in for <Insert system name> piping to verify actual locations of piping connections before [equipment] [fixture] installation.

        - Examine walls, floors, roofs, and <Insert description> for suitable conditions where <Insert product or system> will be installed.

        - [Prepare written report, endorsed by Installer, listing conditions detrimental to performance.]

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

Retain this article only if it supplements Division 01 requirements and includes provisions that apply specifically to individual Sections for cleaning, repairing, protecting, or otherwise preparing substrates for installation.

Repair requirements, which may be appropriate only for remodeling work, pose the risk of implying that repair is a suitable alternative to initially performing work to comply with requirements and of placing the responsibility for repair on Installer rather than on Contractor.

      - ADJUSTING

        - Adjust [hardware] [moving parts] [Insert operable component] to function smoothly, and lubricate as recommended by manufacturer.

        - Occupancy Adjustments:  When requested within [12] <Insert number> months from date of Substantial Completion, provide on-site assistance in adjusting system to suit actual occupied conditions.  Provide up to [two] <Insert number> visits to Project during other-than-normal occupancy hours for this purpose.

Insert "Cleaning" Article for specific requirements for cleaning the completed Work so it functions and performs properly.

      - DEMONSTRATION

        - [Engage a factory-authorized service representative to train] [Train] Owner's maintenance personnel to adjust, operate, and maintain units.

END OF SECTION 08 12 16