SECTION 08 42 29.23 - SLIDING AUTOMATIC ENTRANCES

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes [exterior] [and] [interior], sliding, power-operated automatic entrances.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - [Section 03 30 00 "Cast-in-Place Concrete"] [Section 03 30 53 "Miscellaneous Cast-in-Place Concrete"] for [installing recessed metal frames for control mats in concrete] [and] [forming recesses in concrete for recessed thresholds].

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

        - For automatic door terminology, refer to BHMA A156.10 for definitions of terms.

      - COORDINATION

Retain applicable requirements in this article to suit Project.

        - Coordinate sizes and locations of recesses in concrete floors for [recessed sliding tracks] [and] [recessed control mats] that control automatic entrances.  Concrete, reinforcement, and formwork requirements are specified elsewhere.

        - Templates:  Distribute for doors, frames, and other work specified to be factory prepared for installing automatic entrances.

        - Coordinate hardware with doors, frames, and related work to ensure proper size, thickness, hand, function, and finish.  Coordinate hardware for automatic entrances with hardware required for rest of Project.

        - Electrical System Roughing-in:  Coordinate layout and installation of automatic entrances with connections to power supplies [ and access-control system].

      - PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

        - Preinstallation Conference:  Conduct conference at [Project site] <Insert location>.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data:  For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for automatic entrances.

          - Include rated capacities, operating characteristics, electrical characteristics, and furnished specialties and accessories.

        - Shop Drawings:  For automatic entrances.

If custom components are required and detailed or isometric Shop Drawing are acceptable, insert provisions in a subparagraph below.  If Drawings are insufficient, insert provisions for samples in "Samples for Verification" Paragraph below to indicate details of assembly.

          - Include plans, elevations, sections, hardware mounting heights, and attachment details.

          - Indicate dimensions, weights, loads, required clearances, method of field assembly, components, and location and size of each field connection.

          - Include diagrams for power, signal, and control wiring.

          - Indicate locations of activation and safety devices.

          - Include hardware schedule and indicate hardware types, functions, quantities, and locations.

Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs below for two-stage Samples.

        - Samples for Initial Selection:  For units with factory-applied [color] [and] [metal-clad] finishes.

          - Include Samples of hardware and accessories involving color or finish selection.

If Samples of custom components are required, insert provisions in "Samples for Verification" Paragraph below for Samples to indicate details of assembly.

        - Samples for Verification:  For each type of exposed finish required, in manufacturer's standard sizes.

Retain "Delegated-Design Submittal" Paragraph below if design services have been delegated to Contractor.

        - Delegated-Design Submittal:  For automatic entrances.

      - INFORMATIONAL SUBMITTALS

Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

        - Qualification Data:  For [Installer] [manufacturer] [Certified Inspector].

Retain "Product Certificates" Paragraph below to require submittal of product certificates from manufacturers.  Retain option if applicable to Project.

        - Product Certificates:  For each type of automatic entrance.[  Include emergency-exit features of automatic entrances serving as a required means of egress.]

Generally retain "Product Test Reports" Paragraph below for windborne-debris impact-resistance testing if required.

        - Product Test Reports:  For each type of automatic entrance, for tests performed by a qualified testing agency.

Retain "Field quality-control reports" Paragraph below if Contractor is responsible for field quality-control testing and inspecting.

        - Field quality-control reports.

        - Sample Warranties:  For manufacturer's special warranties.

      - CLOSEOUT SUBMITTALS

        - Operation and Maintenance Data:  For automatic entrances, safety devices, and control systems to include in operation and maintenance manuals.

      - QUALITY ASSURANCE

Retain "Manufacturer Qualifications" Paragraph below if AAADM company certificate is required.  Default manufacturer qualifications are specified in Section 014000 "Quality Requirements."

        - Manufacturer Qualifications:  A manufacturer with company certificate issued by AAADM indicating that manufacturer has a Certified Inspector on staff.

        - Installer Qualifications:  Manufacturer's authorized representative who is trained and approved for installation and maintenance of units required for this Project[ and who employs a Certified Inspector]. Automatic Sliding Door units must be installed by “AAADM” Certified Trained Technicians.  Certification Numbers must be presented prior to commencing work.

Retain "Maintenance Proximity" Subparagraph below if retaining "Maintenance Service" Article.

          - Maintenance Proximity:  Not more than [two] <Insert number> hours' normal travel time from Installer's place of business to Project site.

        - Certified Inspector Qualifications:  Certified by AAADM.

        - Doors shall meet performance design criteria of the following standards:

ANSI/BHMA 156.10

NFPA 101

Underwriters Laboratories UL 325

International Building Code (IBC)

International Conference of Building Officials (ICBO)

Building Officials and Code Administrators International (BOCA)

National Association of Architectural Metal Manufacturers (NAAMM)

American Architectural Manufacturers Association (AAMA)

American Association of Automatic Door Manufacturers (AAADM)

      - WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

        - Special Warranty:  Manufacturer agrees to repair or replace components of automatic entrances that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures including, but not limited to, excessive deflection.

            - Faulty operation of operators, controls, and hardware.

            - Deterioration of metals, metal finishes, and other materials beyond normal weathering and use.

Verify available warranties and warranty periods for units and components.

          - Warranty Period:  Two years from date of Substantial Completion.

Verify available special finish warranties with manufacturers.  Extended 20-year finish warranties are sometimes available for 70 percent fluoropolymer coatings.

        - Special Finish Warranty:  Manufacturer agrees to repair or replace components on which finishes fail in materials or workmanship within specified warranty period.

          - Deterioration includes, but is not limited to, the following:

            - Color fading more than 5 Hunter units when tested according to ASTM D 2244.

            - Chalking in excess of a No. 8 rating when tested according to ASTM D 4214.

            - Cracking, checking, peeling, or failure of paint to adhere to bare metal.

AAMA 2604 represents a five-year set of tests; AAMA 2605 represents a 10-year set.  Warranty periods offered by manufacturers vary depending on finish.  Verify available warranties and warranty periods for units and components.

          - Warranty Period:  [Five] [10] [20] <Insert number> years from date of Substantial Completion.

- PRODUCTS

See Editing Instruction No. 1 in the Evaluations for cautions about named manufacturers and products.  For an explanation of options and Contractor's product selection procedures, see Section 016000 "Product Requirements."

      - AUTOMATIC ENTRANCE ASSEMBLIES

        - Source Limitations:  Obtain sliding automatic entrances from single source from single manufacturer.

        - Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application.

        - Power-Operated Door Standard:  BHMA A156.10.

      - PERFORMANCE REQUIREMENTS

Retain "Delegated Design" Paragraph below if Contractor is required to assume responsibility for design.

        - Delegated Design:  Engage a qualified professional engineer, as defined in Section 01 40 00 "Quality Requirements," to design automatic entrances.

        - Structural Performance:  Automatic entrances shall withstand the effects of gravity loads and the following loads and stresses within limits and under conditions indicated according to [ASCE/SEI 7] <Insert requirement>.

Consult a structural engineer experienced in engineering building entrances of type indicated to quantify design loads applicable to Project.  Verify compliance with codes.

Retain "Seismic Loads" and "Wind Loads" subparagraphs below if design loads and load combinations are not indicated on Drawings.

          - Wind Loads:  <Insert loads>.

In windborne-debris regions of hurricane-prone areas, authorities having jurisdiction may require debris-impact and cyclic-pressure testing for glazed assemblies.  Retain first option in "Windborne-Debris Impact Resistance" Paragraph below for entrances within 30 feet (9.1 m) of grade; retain second option for entrances more than 30 feet (9.1 m) above grade according to the IBC.  See "Windborne Debris" Article in the Evaluations, and verify requirements of authorities having jurisdiction before retaining paragraph below.

        - Windborne-Debris Impact Resistance:  Automatic entrances shall pass [large-missile-impact] [small-missile-impact] and cyclic-pressure tests of [ASTM E 1996 according to the IBC] <Insert testing and code requirements> for [Wind Zone 1] [Wind Zone 2] [Wind Zone 3] [Wind Zone 4].

        - Thermal Movements:  Allow for thermal movements from ambient and surface temperature changes.

          - Temperature Change:  120 deg F (67 deg C), ambient; 180 deg F (100 deg C), material surfaces.

        - Operating Temperature Range:  Automatic entrances shall operate within minus 20 to plus 122 deg F (minus 29 to plus 50 deg C) .

        - Air Infiltration:  Maximum air leakage through fixed glazing and framing areas of 1.25 cfm/sq. ft. (6.4 L/s x sq. m) of fixed entrance-system area when tested according to ASTM E 283 at a minimum static-air-pressure difference of 1.57 lbf/sq. ft. (75 Pa)  .

        - Opening Force:

"Power-Operated Doors" Subparagraph below is according to the IBC and NFPA 101 for power-operated, means-of-egress doors.  The IBC also requires compliance with BHMA A156.10.  Verify requirements of authorities having jurisdiction.

          - Power-Operated Doors:  Not more than 50 lbf (222 N) required to manually set door in motion if power fails, and not more than 15 lbf (67 N) required to open door to minimum required width.

"Breakaway Device for Power-Operated Doors" Subparagraph below is according to BHMA A156.10.

          - Breakaway Device for Power-Operated Doors:  Not more than 50 lbf (222 N) required for a breakaway door or panel to open.

        - Entrapment-Prevention Force:

Requirement in "Power-Operated Sliding Doors" Subparagraph below is from BHMA A156.10.  Verify requirements of authorities having jurisdiction.

          - Power-Operated Sliding Doors:  Not more than 30 lbf (133 N) required to prevent stopped door from closing.

      - SLIDING AUTOMATIC ENTRANCES

        - Assemblies Package:  The manufacturer’s sliding door package shall consist of the following materials in order to make a complete package installation: framing, flush mounted header (mounted between jambs), sliding door panel(s), stationary panel(s), operators (belt drive only linear rod not accepted), activation and safety devices, carrier assemblies, noise isolating roller track, threshold, and guide tracks (to match threshold dimensions on full breakout units).

Copy Sliding Automatic Entrance" Paragraph below and re-edit for each product.

Insert drawing designation for each product required.  Use these designations on Drawings to identify each product.

        - [Sliding] Automatic Entrance <Insert drawing designation>:

          - Basis-of-Design Product:  Subject to compliance with requirements, provide 5100 series sliding entrances by Record USA, or comparable product by one of the following:

            - [Single-] [and] [Biparting-]Sliding Units:

              - Nabco Entrances Inc/Gyro-Tech.

              - Gildor Automatic Doors.

              - <Substitutions:  See Section 01 25 00 - Substitution Procedures.

        - Acceptable Configurations:

- Full Breakout: Sliding and stationary sidelite panel(s) ALL swing clear for means of emergency egress.

- Fixed sidelite: Only sliding panel swings away for emergency egress.

- Bi-folding automatic doors and “all-glass” automatic doors are not allowed.

- Operator Features

Revise list below to suit Project.  Consult manufacturers for availability and necessity of adjustability for specific applications.

            - Power opening and closing.

            - Drive System:  [belt].

            - Adjustable opening and closing speeds.

            - Adjustable hold-open time between zero and 30 seconds.

            - Obstruction recycle.

            - On-off/hold-open switch to control electric power to operator[, key operated].

            - <Insert features required>.

          - Sliding-Door Carrier Assemblies and Overhead Roller Tracks:  Carrier assembly that allows vertical adjustment; consisting of nylon- or delrin-covered, ball-bearing-center steel wheels operating on a continuous roller track, or ball-bearing-center steel wheels operating on a nylon- or delrin-covered, continuous roller track.  Support doors from carrier assembly by cantilever and pivot assembly.

            - Rollers:  Minimum of two ball-bearing roller wheels and two antirise rollers for each active leaf.  Each supporting roller shall be 1-3/4” in diameter.

          - Sliding-Door Threshold:  Threshold members and bottom-guide-track system with stainless-steel, ball-bearing-center roller wheels.

Retain one of two "Configuration" subparagraphs below, or delete both and indicate sill configuration on Drawings.  First subparagraph may be used with or without surface or recessed control mats.  Note Recessed condition hold Coordinate with control system used.

            - Configuration:  Saddle-type threshold across door opening and [surface-mounted] [recessed] guide-track system at sidelites.

            - Configuration:  No threshold across door opening and [surface-mounted](preferred) [recessed] (upon approval) guide-track system at sidelites.

          - Controls:  Activation and safety devices [as indicated on Drawings and ] according to BHMA standards.

Retain option in "Controls" Subparagraph above or retain one activation device and one safety device from three "Activation Device" and four "Safety Device" subparagraphs below.  Consult manufacturers for recommendations; revise to suit Project.

Motion sensor is the most common activation device, and wall push-plate switch is often used for clean rooms.  Key switch is usually a supplementary activation device.

            - Activation Device:  Motion sensor mounted on each side of door header to detect pedestrians in activating zone and to open door.

            - Activation Device:  Control mat installed on ingress side of door to detect pedestrians in activating zone and to open door.

            - Activation Device:  [Push-plate switch] [Push-button switch] [Key switch][ on each side of door] to activate door operator.

Retain one "Safety Device" Subparagraph below with any activation device above.

            - Safety Device:  Two photoelectric beams mounted in sidelite jambs on each side of door to detect pedestrians in presence zone and to prevent door from closing.

            - Safety Device:  Presence sensor mounted to underside of door header and two photoelectric beams mounted in sidelite jambs on one side of the door to detect pedestrians in presence zone and to prevent door from closing.

            - Safety Device:  Presence sensor mounted on each side of door header and two photoelectric beams mounted in sidelite jambs on one side of the door to detect pedestrians in presence zone and to prevent door from closing.

Safety control mats in "Safety Device" Subparagraph below are typically used only with activation control mats.

            - Safety Device:  Control mat(s) installed on egress side of door to detect pedestrians in presence and safety zones and to prevent door from closing.

Presence sensor in "Sidelite Safety Device" Subparagraph below is safety device offered by some manufacturers; it is optional feature and in addition to other safety devices.  Retain if required.

            - Sidelite Safety Device:  Presence sensor, mounted above each sidelite on side of door opening through which doors travel, to detect obstructions and to prevent door from opening.

Retain "Opening-Width Control" Subparagraph below only for exterior locations or where reduced opening-width control is required to conserve energy and reduce amount of air exchange.

            - Opening-Width Control:  Two-position switch that in the normal position allows sliding doors to travel to full opening width and in the alternate position reduces opening to a selected partial opening width.

Retain "Finish" or "Metal Cladding and Finish" Subparagraph below.

Revise "Finish" Subparagraph below if different finish is required for components of automatic entrances.  If more than one finish is required, indicate location of each on Drawings, in schedules, or by inserts.

          - Finish:  Finish framing, door(s), and header with [Class I, clear anodic finish] [Class II, clear anodic finish] [Class I, color anodic finish] [Class II, color anodic finish] [baked-enamel or powder-coat finish] [high-performance organic finish (two-coat fluoropolymer)] [high-performance organic finish (three-coat fluoropolymer)] [finish matching adjacent curtain wall] [finish matching adjacent storefront].

Delete "Color" Subparagraph below only for clear anodic finishes.  Color anodic finishes may vary in color range and availability among manufacturers.

            - Color:  [Light bronze] [Medium bronze] [Dark bronze] [Black] [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors and color densities] <Insert color>.

Retain "Metal Cladding and Finish" Subparagraph below only for metal-clad automatic entrances.  Verify availability of metal cladding with manufacturers before specifying.  Revise subparagraph if different finish is required for components of automatic entrances.  If more than one finish is required, indicate location of each on Drawings, in schedules, or by inserts.

          - Metal Cladding and Finish:  Clad framing, door(s), and header with [No. 4 directional-satin-finish stainless-steel sheet] [No. 8 mirrorlike reflective, nondirectional-polish-finish stainless-steel sheet] [satin-brass sheet] [polished-brass sheet] [satin-bronze sheet] [polished-bronze sheet] [metal sheet in finish matching Architect's sample] [metal sheet in finish as selected by Architect from manufacturer's full range] [metal sheet in finish matching adjacent storefront] <Insert finish>.

      - ENTRANCE COMPONENTS

        - Framing Members:  Extruded aluminum, minimum 0.125 inch (3.2 mm) thick and reinforced as required to support imposed loads.

          - Nominal Size:  [As indicated on Drawings] [1-3/4 by 4-1/2 inches (45 by 115 mm)] [1-3/4 by 6 inches (45 by 150 mm)] <Insert dimensions>.

Retain "Extruded Glazing Stops and Applied Trim" Subparagraph below for separately framed sidelites or transoms.

          - Extruded Glazing Stops and Applied Trim:  Minimum 0.062-inch (1.6-mm) wall thickness.

Retain "Stile and Rail Doors" Paragraph below for stile and rail doors.

        - Stile and Rail Doors:  1-3/4-inch- (45-mm-) thick, glazed doors with minimum 0.125-inch- (3.2-mm-) thick, extruded-aluminum tubular stile and rail members.  Mechanically fasten corners with reinforcing brackets that are welded, or incorporate concealed tie-rods that span full length of top and bottom rails.

          - Glazing Stops and Gaskets:  [Beveled] [Square], snap-on, extruded-aluminum stops and preformed gaskets.

Verify, with manufacturers, availability of stile and rail dimensions in "Stile Design" and "Rail Design" subparagraphs below.

          - Stile Design:  [As indicated on Drawings] [Thin stile, less than 1-3/4-inch (45-mm) nominal width] [Narrow stile, 2-1/8-inch (55-mm) nominal width] [Medium stile, 3-1/2-inch (90-mm) nominal width] [Wide stile, more than 4-inch (100-mm) nominal width].

Last option in "Rail Design" Subparagraph below is required for manual doors along an accessible route but is not required for automatic doors.

          - Rail Design:  [As indicated on Drawings] [5-inch (125-mm) nominal height] [6-1/2-inch (165-mm) nominal height] [10-inch (254-mm) nominal height].

Retain "Muntin Bars" Subparagraph below if required.  Many manufacturers include muntins as standard.

          - Muntin Bars:  Horizontal tubular rail member for each door; match stile design and finish.

        - [Sidelite(s)] [and] [Transom]:  1-3/4-inch- (45-mm-) deep [sidelite(s)] [and] [transom] with minimum 0.125-inch- (3.2-mm-) thick, extruded-aluminum tubular stile and rail members matching door design.

Retain one of two "Glazing Stops and Gaskets" subparagraphs below; second is for all-glass sliding doors.

          - Glazing Stops and Gaskets:  Same materials and design as for stile and rail door.

          - Glazing Stops and Gaskets:  [Beveled] [Square], snap-on, extruded-aluminum stops and preformed gaskets.

Retain "Muntin Bars" Subparagraph below if required for sidelites.

          - Muntin Bars:  Horizontal tubular rail members for each sidelite; match stile design.

        - Headers:  Fabricated from minimum 0.125-inch- (3.2-mm-) thick extruded aluminum and extending full width of automatic entrance units to conceal door operators and controls.  Provide hinged or removable access panels for service and adjustment of door operators and controls.  Secure panels to prevent unauthorized access.

          - Mounting:  [Surface mounted] [Concealed, with one side of header flush with framing].

          - Capacity:  Capable of supporting doors up to [175 lb (79 kg) per leaf over spans up to 14 feet (4.3 m)] <Insert load and span required> without intermediate supports.

            - Provide sag rods for spans exceeding 14 feet (4.3 m).

        - Brackets and Reinforcements:  High-strength aluminum with nonstaining, nonferrous shims for aligning system components.

        - Signage:  As required by cited BHMA standard.

First option in "Application Process" Subparagraph below is most common.

          - Application Process:  [Decals] [Silk-screened] [Door manufacturer's standard process] <Insert requirement>.

Retain subparagraph below for signs on field-installed glass surfaces.

          - Provide sign materials with instructions for field application after glazing is installed.

      - MATERIALS

        - Aluminum:  Alloy and temper recommended by manufacturer for type of use and finish indicated.

          - Extrusions:  ASTM B 221 (ASTM B 221M).

          - Sheet:  ASTM B 209 (ASTM B 209M).

        - Steel Reinforcement:  Reinforcement with corrosion-resistant primer complying with SSPC-PS Guide No. 12.00 applied immediately after surface preparation and pretreatment.  Use surface preparation methods according to recommendations in SSPC-SP COM and prepare surfaces according to applicable SSPC standard.

        - Stainless-Steel Bars:  ASTM A 276 or ASTM A 666, [Type 304] [Type 316] <Insert type>.

        - Stainless-Steel Tubing:  ASTM A 554, [Grade MT 304] [Grade MT 316] <Insert grade>.

Retain one or more of "Stainless-Steel Sheet," "Brass Sheet," and "Bronze Sheet" paragraphs below for metal-clad finishes.

        - Stainless-Steel Sheet:  ASTM A 240/A 240M or ASTM A 666, [Type 304] [Type 316] <Insert type>, stretcher-leveled standard of flatness, in entrance manufacturer's standard thickness.

Retain "Brass Sheet" Paragraph below for brassy yellow color; revise to suit Project.

        - Brass Sheet:  ASTM B 36/B 36M, Alloy UNS No. C26000 (cartridge brass, 70 percent copper), in entrance manufacturer's standard thickness.

Retain "Bronze Sheet" Paragraph below for bronze look.  Neither alloy is a true tin bronze, but both closely match color of extruded architectural bronze (also not a true tin bronze).  Revise if only one of these two alloys is acceptable or to specify another alloy.

        - Bronze Sheet:  ASTM B 36/B 36M, Alloy UNS No. C28000 (muntz metal, 60 percent copper) or Alloy UNS No. C23000 (red brass, 85 percent copper), in entrance manufacturer's standard thickness.

Retain "Expanded Aluminum Mesh" or "Polycarbonate Sheet" Paragraph below for guide-rail infill panel if required.  Revise to suit Project or insert another material if required.

Options in "Expanded Aluminum Mesh" Paragraph below are types included in ASTM F 1267.

        - Expanded Aluminum Mesh:  [Expanded] [Expanded and flattened] aluminum sheet according to the geometry of ASTM F 1267.

        - Polycarbonate Sheet:  ASTM C 1349, Appendix X1, Type II, coated, mar-resistant, UV-stabilized polycarbonate with coating on both surfaces.

        - Glazing:  As specified in [Section 08 80 00 "Glazing."] [Section 08 88 53 "Security Glazing."]

        - Sealants and Joint Fillers:  As specified in Section 07 92 00 "Joint Sealants."

Retain "Nonmetallic, Shrinkage-Resistant Grout" Paragraph below for leveling floor tracks.

        - Nonmetallic, Shrinkage-Resistant Grout:  Premixed, nonmetallic, noncorrosive, nonstaining grout; complying with ASTM C 1107/C 1107M; of consistency suitable for application.

        - Bituminous Coating:  Cold-applied asphalt emulsion complying with ASTM D 1187.

        - Fasteners and Accessories:  Corrosion-resistant, nonstaining, nonbleeding fasteners and accessories compatible with adjacent materials.

      - DOOR OPERATORS AND CONTROLS

        - General:  Provide operators and controls, which include activation and safety devices, according to BHMA standards, for condition of exposure, and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.

        - Door Operators:  Provide door operators of size recommended by manufacturer for door size, weight, and movement.

          - Door Operator Performance:  Door operators shall open and close doors and maintain them in fully closed position when subjected to Project's design wind loads.

          - Electromechanical Operators: Door movement shall be driven by a sealed, low voltage class II, 1/8 horsepower 24v DC motor and gearbox and nylon reinforced drive belt.  The motor shall have a current draw of not more than 100 watts.  The sealed motor gearbox assembly shall be capable of driving door leaves up to 225 lbs.  A second motor gearbox can be utilized on the same application giving capability of moving door panels weighing up to 450 lbs.  The motor gearbox assembly shall be mounted directly to the header extrusion by means of three (3) each M5 x ¼” threaded standoff bolts.

        - Motion Sensors:  Self-contained, K-band-frequency, microwave-scanner units; fully enclosed by its plastic housing; adjustable to provide detection-field sizes and functions required by BHMA A156.10.

          - Provide capability for switching between bidirectional and unidirectional detection.

          - For one-way traffic, sensor on egress side shall not be active when doors are fully closed.

          - Approved Sensors (for sliding doors):

            - Wizard by BEA

            - Eye One by Optex

        - Presence Sensors:  Self-contained, active-infrared scanner units; adjustable to provide detection-field sizes and functions required by BHMA A156.10.  Sensors shall remain active at all times.

          - Approved Sensors):

            - Wizard by BEA

            - Eye One by Optex

        - Photoelectric Beams:  Pulsed infrared, sender-receiver assembly for recessed mounting.  Beams shall not be active when doors are fully closed.

Control mats and activation switches in "Control Mats," "Push-Plate Switch," "Push-Button Switch," "Key Switch," and "Wireless or Remote Radio Control Switch" paragraphs below vary among manufacturers.  Consult manufacturers and revise retained devices as needed.

        - Control Mats:  1/2-inch- (13-mm-) thick, synthetic-rubber or flexible-plastic mat in safety-ribbed surface pattern, with extruded-aluminum frame; with pressure switches for low-voltage control wiring; and complying with performance requirements of BHMA A156.10.

          - Frame:  [Recessed to fit flush with floor, with concealed anchors] [Surface mounted, with tapered safety edge].

          - Size:  As indicated, but no smaller than required by BHMA A156.10 including Appendix A.

If needed, insert requirements for monograms and directional arrows.

          - Color:  [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from full range of industry colors and color densities].

        - Push-Plate Switch:  Momentary-contact door-control switch with flat push-plate actuator[ with contrasting-colored, engraved message].

Retain one of two "Configuration" subparagraphs below.  Insert mounting height for wall switches if not indicated on Drawings.

          - Configuration:  [Round] [Square] push plate with 4-by-4-inch (100-by-100-mm) junction box.

            - Mounting:  [As indicated on Drawings] [Recess mounted, semiflush in wall] [Surface mounted on wall].

          - Configuration:  Rectangular push plate with 2-by-4-inch (50-by-100-mm) junction box.

            - Mounting:  [As indicated on Drawings] [Recess mounted, semiflush in wall] [Recess mounted in door jamb] [Surface mounted on wall] [Surface mounted on post] [Surface mounted on guide rail].

          - Push-Plate Material:  [Stainless steel] [Plastic] as selected by Architect from manufacturer's full range.

          - Message:  [Plain face with no message.] ["Push to Open."] [International symbol of accessibility.] [International symbol of accessibility and "Push to Open."]

        - Push-Button Switch:  Momentary-contact door-control switch with one red-button actuator; enclosed in nominal [2-by-4-inch (50-by-100-mm)] [4-by-4-inch (100-by-100-mm)] junction box.

          - Provide faceplate engraved with "Press to Open" letters[ and international symbol of accessibility] in contrasting color.

          - Provide blue plastic cover engraved with "Press Button to Open" in white letters and international symbol of accessibility.

          - Mounting:  [As indicated on Drawings] [Surface mounted on wall] [Surface mounted on post] [Surface mounted on guide rail] [Recess mounted in wall].

          - Face-Plate Material:  [Stainless steel] [Painted metal] as selected by Architect from manufacturer's full range.

        - Key Switch:  Recess-mounted, door-control switch with key-controlled actuator; enclosed in 2-by-4-inch (50-by-100-mm) junction box.  Provide faceplate engraved with letters indicating switch functions.

          - Face-Plate Material:  [Stainless steel] [Painted metal] as selected by Architect from manufacturer's full range.

          - Functions:  [On-off, momentary contact] [On-off, maintained contact] [Two-way automatic, hold open, one-way exit, and off] [Two-way automatic, hold open, one-way exit, off, full open, and partial open].

          - Mounting:  [As indicated on Drawings] [Recess mounted, semiflush in wall] [Recess mounted in door jamb] [Surface mounted on wall] [Surface mounted on post].

        - Wireless or Remote Radio Control Switch:  Auxiliary radio control system consisting of header-mounted receiver and [wall-mounted] [hand-held, battery-operated] transmitter switch[ for each entrance] <Insert requirement>.

          - Wall-Mounted Transmitter Switch:  One red-button, momentary-contact actuator enclosed in 4-by-4-inch (100-by-100-mm) junction box.  Provide blue plastic cover engraved with "Press Button to Open" in white letters and international symbol of accessibility.

If required, insert another type of sensing device, switch, or access-control device.  Verify availability with manufacturers.

        - Electrical Interlocks:  Unless units are equipped with self-protecting devices or circuits, provide electrical interlocks to prevent activation of operator when door is locked, latched, or bolted.

      - HARDWARE

        - Weather Stripping:  Replaceable components.

          - Sliding Type:  AAMA 701, made of wool, polypropylene, or nylon woven pile with nylon-fabric or aluminum-strip backing.

        - ANSI A156.5, Grade 1, 2-Point Locking provided and installed in the strike rail. Mfg’s. standard hook bolt lock operated by exterior cylinder and interior cylinder

- Hook Bolt Latch: Laminated steel, latching into jamb or strike rail

- Two-Point Locking: Provide locking device that provide locking capability into the adjacent strike rail or jamb and extends a flush bolt into the overhead carriage assembly. Deadbolt locking options include the following:

  - Three-Point Locking for bi-parting door

  - Lock indicators

  - Adams-Rite 4550

        - Reset Button: To be installed at the top of the Automatic door header.  Keyed Junction Box: Required to kill power to door, must use cylinder with IC Best 7 pin core to be mounted no higher than (5) foot from the finish floor.

        - Flush Panic Exit Device recessed in muntin bar or as specified for each project.

        - There must be (20) keys included for the (new) “Automatic” sliding door units.

        - S.M.A.R.T. Panel (Self Monitoring Accurate Reporting Technology): Provide manufacturer’s standard jamb mounted control panel for complete control and reporting of the automatic sliding door. Control panel capabilities include, but are not limited to the following:

- Power On/Off

- Full Open/Partial Open

- Hold Open/Closed/Automatic Operation

- Daily Safety Check Reminder

- Diagnostic Reporting and Door Cycle Count

        - Weather Stripping:  Weather Stripping shall be along the perimeter all door panels and sidelites to reduce energy loss. Standard weather stripping includes the following:

- Adjustable nylon sweep in the bottom of sliding door(s)

- Double pile weather stripping on the strike rail of sliding door(s)

- Single pile weather stripping at the following locations:

  - Between the carriage assembly and header

  - Lead stile of sidelite(s)

  - Pivot stile of sidelite(s)

  - Rain Guard – To be installed at all “Outside” mounted sensors.

      - ACCESSORIES

Retain one of two "Guide Rails" paragraphs below if guide rails are required.  BHMA and IBC do not require guide rails for sliding doors.  However, guide rails might still be useful in some door layout conditions and where control mats are used.

        - Guide Rails:  [Anodized aluminum] [Baked-enamel or powder-coated aluminum] [Stainless steel], fabricated from [bars] [or] [tubing], minimum 30 inches (762 mm) high, and finished to match doors unless otherwise indicated; positioned and projecting from face of door jamb for distance as indicated, but not less than [that required by BHMA A156.10 for type of door and direction of travel] <Insert dimension>; with filler panel.

          - Filler Panel:  [Expanded aluminum mesh] [Clear polycarbonate sheet] [Colored polycarbonate sheet] <Insert material>.

Retain first subparagraph below if required; delete if shown on Drawings.

            - Orient expanded aluminum mesh with long dimension of diamonds [parallel to top rail] [perpendicular to top rail].

            - Color:  [Match Architect's sample] [As selected by Architect from manufacturer's full range] <Insert color>.

          - Mounting:  [As indicated on Drawings] [Jamb and floor] [Floor, freestanding].

Retain "Aluminum Finish" or "Stainless-Steel Finish" Subparagraph below; revise to suit Project.

          - Aluminum Finish:  [Class I, clear anodic finish] [Class II, clear anodic finish] [Class I, color anodic finish] [Class II, color anodic finish] [Baked-enamel or powder-coat finish] [Finish matching door and frame] <Insert finish>.

Delete "Color" Subparagraph below only for clear anodic finishes.  Color anodic finishes may vary in color range and availability among manufacturers.

            - Color:  [Light bronze] [Medium bronze] [Dark bronze] [Black] [Match Architect's sample] [As selected by Architect from full range of industry colors and color densities] <Insert color>.

          - Stainless-Steel Finish:  [No. 4 directional-satin-finish stainless steel] [Finish matching door and frame] <Insert finish>.

If retaining "Guide Rails" Paragraph below, verify that guide-rail design and layout complies with BHMA A156.10.

        - Guide Rails:  See [Section 05 52 13 "Pipe and Tube Railings."] [Section 05 73 00 "Decorative Metal Railings."]

      - FABRICATION

        - General:  Factory fabricate automatic entrance components to designs, sizes, and thicknesses indicated and to comply with indicated standards.

          - Form aluminum shapes before finishing.

          - Weld in concealed locations to greatest extent possible to minimize distortion or discoloration of finish.  Remove weld spatter and welding oxides from exposed surfaces by descaling or grinding.

          - Use concealed fasteners to greatest extent possible.  Where exposed fasteners are required, use countersunk Phillips flat-head machine screws[, finished to match framing] [, fabricated from stainless steel].

            - Where fasteners are subject to loosening or turning out from thermal and structural movements, wind loads, or vibration, use self-locking devices.

            - Reinforce members as required to receive fastener threads.

          - Where aluminum will contact dissimilar metals, protect against galvanic action by painting contact surfaces with primer or by applying sealant or tape recommended by manufacturer for this purpose.

        - Framing:  Provide automatic entrances as prefabricated assemblies.  Complete fabrication, assembly, finishing, hardware application, and other work before shipment to Project site.

          - Fabricate tubular and channel frame assemblies with welded or mechanical joints.  Provide subframes and reinforcement as required for a complete system to support required loads.

          - Perform fabrication operations in manner that prevents damage to exposed finish surfaces.

          - Form profiles that are sharp, straight, and free of defects or deformations.

          - Provide components with concealed fasteners and anchor and connection devices.

          - Fabricate components with accurately fitted joints with ends coped or mitered to produce hairline joints free of burrs and distortion.

          - Fabricate exterior components to drain condensation and water passing joints within system to the exterior.

          - Provide anchorage and alignment brackets for concealed support of assembly from building structure.

          - Allow for thermal expansion of exterior units.

        - Doors:  Factory fabricated and assembled in profiles indicated.  Reinforce as required to support imposed loads and for installing hardware.

Retain "Metal Cladding" Paragraph below only for metal-clad finishes.

        - Metal Cladding:  Factory-fabricated and installed metal cladding, completely covering all visible surfaces as part of prefabricated entrance assembly before shipment to Project site.

          - Perform fabrication operations in manner that prevents damage to exposed finish surfaces.

          - Form profiles that are sharp, straight, and free of defects or deformations.

          - Provide components with concealed fasteners and anchor and connection devices.

          - Fabricate components with accurately fitted joints with ends coped or mitered to produce hairline joints free of burrs and distortion.

          - Fabricate exterior components to drain water passing joints and condensation and moisture occurring or migrating within system to the exterior.

          - Allow for thermal expansion at exterior entrances.

        - Door Operators:  Factory fabricated and installed in headers, including adjusting and testing.

        - Glazing:  Fabricate framing with minimum glazing edge clearances for thickness and type of glazing indicated, according to GANA's "Glazing Manual."

          - Doors and Sidelites: Doors and sidelites shall be capable of accepting the same thickness of glass for fixed sidelite and full break-out units. Exterior Glass Stop Extrusion: Doors and side-lites shall be provided with non-removable security glazing system at exterior.  Glazing Prep: Standard glazing prep to be for (¼”) glass, optional glazing prep includes: (1”) glazing & (5/8”) glazing prep.

Insert requirements for factory glazing if available from manufacturers.  Automatic entrances are generally field glazed.

        - Hardware:  Factory install hardware to greatest extent possible; remove only as required for final finishing operation and for delivery to and installation at Project site.  Cut, drill, and tap for factory-installed hardware before applying finishes.

          - Provide sliding-type weather stripping, mortised into door, at perimeter of doors[ and breakaway sidelites].

        - Controls:

          - General:  Factory install activation and safety devices in doors and headers as required by BHMA A156.10 for type of door and direction of travel.

          - Install photoelectric beams in vertical jambs of sidelites, with dimension above finished floor as follows:

            - Top Beam:  [48 inches (1219 mm)] <Insert dimension>.

            - Bottom Beam:  [24 inches (610 mm)] <Insert dimension>.

      - GENERAL FINISH REQUIREMENTS

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

Retain first paragraph below for coatings and anodic finishes.

        - Apply organic and anodic finishes to formed metal after fabrication unless otherwise indicated.

        - Appearance of Finished Work:  Noticeable variations in same piece are not acceptable.  Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - ALUMINUM FINISHES

If retaining more than one finish in paragraphs below, indicate location of each on Drawings or by inserts.

Retain one of two options in "Clear Anodic Finish" Paragraph below.  Verify availability with manufacturers.

        - Clear Anodic Finish:  AAMA 611, [AA-M12C22A41, Class I, 0.018 mm] [AA-M12C22A31, Class II, 0.010 mm] or thicker.

Retain one of two options in "Color Anodic Finish" Paragraph below.  Verify availability with manufacturers.

        - Color Anodic Finish:  AAMA 611, [AA-M12C22A42/A44, Class I, 0.018 mm] [AA-M12C22A32/A34, Class II, 0.010 mm] or thicker.

"Baked-Enamel or Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

        - Baked-Enamel or Powder-Coat Finish:  AAMA 2603 except with a minimum dry film thickness of 1.5 mils (0.04 mm).  Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

Retain one of two "High-Performance Organic Finish" paragraphs below; if both are required, indicate location of each system on Drawings, in schedules, or by inserts.  Retain AAMA 2604 or AAMA 2605 for high- or superior-performance organic coatings, respectively, on extrusions and panels.  If specific products are required, name coating manufacturers and products.

        - High-Performance Organic Finish:  Two-coat fluoropolymer finish complying with [AAMA 2604] [AAMA 2605] and containing not less than [50] [70] percent PVDF resin by weight in color coat.  Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

        - High-Performance Organic Finish:  Three-coat fluoropolymer finish complying with AAMA 2605 and containing not less than [50] [70] percent PVDF resin by weight in both color coat and clear topcoat.  Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

- EXECUTION

      - EXAMINATION

        - Examine conditions, with Installer present, for compliance with requirements for installation tolerances, header support, and other conditions affecting performance of automatic entrances.

        - Examine roughing-in for electrical systems to verify actual locations of power connections before automatic entrance installation.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - General:  Install automatic entrances according to manufacturer's written instructions and cited BHMA standard for direction of pedestrian travel, including signage, controls, wiring, and connection to the building's power supply.

          - Do not install damaged components.  Fit frame joints to produce hairline joints free of burrs and distortion.  Rigidly secure nonmovement joints.  Seal joints watertight.

          - Where aluminum will contact dissimilar metals, protect against galvanic action by painting contact surfaces with primer or by applying sealant or tape recommended by manufacturer for this purpose.

          - Where aluminum will contact concrete or masonry, protect against corrosion by painting contact surfaces with bituminous coating.

        - Entrances:  Install automatic entrances plumb and true in alignment with established lines and grades without warp or rack of framing members and doors.  Anchor securely in place.

Retain first subparagraph below for field-installed hardware items.

          - Install surface-mounted hardware using concealed fasteners to greatest extent possible.

          - Set headers, carrier assemblies, tracks, operating brackets, and guides level and true to location with anchorage for permanent support.

          - Install components to drain water passing joints, condensation occurring within framing members, and moisture migrating within system to exterior.

          - Level recesses for recessed thresholds using nonshrink grout.

        - Door Operators:  Connect door operators to electrical power distribution system.

Retain "Access-Control Devices" Paragraph below only if access-control devices are inserted in Part 2.

        - Access-Control Devices:  Connect access-control devices to access-control system as specified in Section 28 13 00 "Access Control."

        - Controls:  Install and adjust activation and safety devices according to manufacturer's written instructions and cited BHMA standard for direction of pedestrian travel.  Connect control wiring according to Section 26 05 19 "Low-Voltage Electrical Power Conductors and Cables."

Retain "Guide Rails" Paragraph below if specifying guide rails in Part 2.  Delete if guide rails are specified in another Section.

        - Guide Rails:  Install rails according to BHMA A156.10, including Appendix A, and manufacturer's written instructions unless otherwise indicated.

Retain "Glazing" Paragraph below unless factory glazing is required in Part 2.

        - Glazing:  Install glazing as specified in [Section 08 80 00 "Glazing."] [Section 08 88 53 "Security Glazing."]

        - Sealants:  Comply with requirements specified in Section 07 92 00 "Joint Sealants" to provide weathertight installation.

          - Set [thresholds, ] [bottom-guide-track system, ]framing members and flashings in full sealant bed.

          - Seal perimeter of framing members with sealant.

        - Signage:  Apply signage on both sides of each door[ and breakaway sidelite] as required by cited BHMA standard for direction of pedestrian travel.

        - Wiring within Automatic Entrance Enclosures:  Bundle, lace, and train conductors to terminal points with no excess and without exceeding manufacturer's written limitations on bending radii.  Provide and use lacing bars and distribution spools.

      - FIELD QUALITY CONTROL

- Automatic Sliding Door units must be installed by “AAADM” Certified Trained Technicians. Certification Numbers must be presented prior to commencing work.

- Installing company of the equipment, to provide local central dispatch system for warranty service, this is to be available 24 hours a day, 365 days per year. A sticker will be placed in a prominent position on the header of each installed unit giving details of local service company, name and telephone number.

- The door company information consisting of the product “Warranty” is required at the time the project is closed out. The “Warranty” document will designate the completion date and the duration of the “Warranty” for work performed.

- Automatic entrances will be considered defective if they do not pass tests and inspections.

- Prepare test and inspection reports.

      - ADJUSTING

        - Adjust hardware, moving parts, door operators, and controls to function smoothly, and lubricate as recommended by manufacturer; comply with requirements of applicable BHMA standards.

Retain subparagraph below for exterior doors.

          - Adjust exterior doors for weathertight closure.

        - Readjust door operators and controls after repeated operation of completed installation equivalent to three days' use by normal traffic (100 to 300 cycles).

        - Occupancy Adjustments:  When requested within [12] <Insert number> months of date of Substantial Completion, provide on-site assistance in adjusting system to suit actual occupied conditions.  Provide up to [two] <Insert number> visits to Project during other-than-normal occupancy hours for this purpose.

      - CLEANING

        - Clean glass and metal surfaces promptly after installation.  Remove excess glazing and sealant compounds, dirt, and other substances.  Repair damaged finish to match original finish.

          - Comply with requirements in [Section 08 80 00 "Glazing"] [for cleaning and maintaining glass.

      - MAINTENANCE SERVICE

Verify with Owner that maintenance service is required for project.

        - Initial Maintenance Service:  Beginning at Substantial Completion, maintenance service shall include [24] months' full maintenance by skilled employees of automatic entrance Installer.  Include [monthly] [quarterly] preventive maintenance, repair or replacement of worn or defective components, lubrication, cleaning, and adjusting as required for proper automatic entrance operation.  Parts and supplies shall be manufacturer's authorized replacement parts and supplies.

          - Engage a Certified Inspector to perform safety inspection after each adjustment or repair and at end of maintenance period.  Furnish completed inspection reports to Owner.

          - Include 24-hour-per-day, 7-day-per-week, emergency callback service.  A sticker will be placed in a prominent position on the header of each installed unit giving details of local service company, name and telephone number.

      - DEMONSTRATION

        - Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain automatic entrances.

END OF SECTION 08 42 29.23