Copyright 2010 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 10 21 16 - SHOWER AND DRESSING COMPARTMENTS

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Shower compartments fabricated from [CMU block with cultured marble, granite, or other solid surface panel (preferred)] [stainless steel] [solid phenolic]

          - Dressing compartments fabricated from [stainless steel] [solid phenolic]

        - Related Sections:

Retain Sections in subparagraphs below that contain requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 05 50 00 "Metal Fabrications" for supports that attach floor-and-ceiling-anchored compartments to overhead structural system.

Wood may provide adequate overhead support for floor-and-ceiling-anchored compartments; delete last option in first subparagraph below if retaining subparagraph above.  Consult manufacturers for recommendations.

          - [Section 06 10 00 "Rough Carpentry"] [Section 06 10 53 "Miscellaneous Rough Carpentry"] for [blocking] [and] [overhead support of floor-and-ceiling-anchored compartments].

Retain first subparagraph below unless accessories are inserted in this Section.  Some manufacturers offer a limited number of standard accessory items.  Verify availability with manufacturers.

          - Section 10 28 00 "Toilet, Bath, and Laundry Accessories" for grab bars, purse shelves, and similar accessories.

      - ACTION SUBMITTALS

        - Product Data:  For each type of product indicated.  Include construction details, material descriptions, dimensions of individual components and profiles, and finishes.

        - LEED Submittals (Projects authorized for LEED certification only)

"Product Data for Credit IEQ 4.4" Subparagraph below applies to LEED-NC, LEED-CI, and LEED-CS.

          - Product Data for Credit IEQ 4.4:  For particleboard, documentation indicating that product contains no urea formaldehyde.

"Laboratory Test Reports for Credit IEQ 4" Subparagraph below applies to LEED for Schools.

          - Laboratory Test Reports for Credit IEQ 4:  For [adhesives] [and] [composite wood products], documentation indicating that products comply with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

        - Shop Drawings:  For shower and dressing compartments.  Include plans, elevations, sections, details, and attachments to other work.

Retain first subparagraph below if required, or revise to suit Project.

          - Show locations of cutouts for compartment-mounted accessories.

Retain first subparagraph below if required for metal compartments; see "Metal Compartments, General" Article in the Evaluations.

          - Show locations of reinforcements for compartment-mounted grab bars.

          - Show locations of centerlines of drains.

Retain subparagraph below for floor-and-ceiling-anchored compartments.

          - Show[ ceiling grid and] overhead support or bracing locations.

        - Samples for Initial Selection:  For each type of compartment indicated.  Include Samples of hardware and accessories for material and color selection.

Delete "Samples for Initial Selection" Paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain paragraph below with or without above.

        - Samples for Verification:  For the following products, in manufacturer's standard sizes unless otherwise indicated:

          - Each type of material, color, and finish required for compartments, prepared on 6-inch- (152-mm-) square Samples of same thickness and material indicated for the Work.

          - Each type of hardware and accessory.

          - Curtain Fabric:  12-inch- (305-mm-) square swatch or larger as required to show complete pattern repeat, from dye lot used for the Work, with specified treatments applied.  Mark top and face of material.

      - INFORMATIONAL SUBMITTALS

Retain paragraph below for product certificates from manufacturers.

        - Product Certificates:  For each type of shower and dressing compartment, from manufacturer.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data:  For shower and dressing compartments to include in maintenance manuals.

      - QUALITY ASSURANCE

Retain first paragraph below if Owner or authorities having jurisdiction require that shower and dressing compartments made of combustible materials comply with surface-burning requirements.  Metal compartments are inherently noncombustible.  Options for flame-spread index correspond to flame-spread- and smoke-developed-index Classes A, B, and C; verify availability of suitable compartments with manufacturers.  See "Fire-Test-Response Characteristics" Article in the Evaluations.

        - Surface-Burning Characteristics:  As determined by testing identical products according to ASTM E 84, or another standard acceptable to authorities having jurisdiction, by a qualified testing agency.  Identify products with appropriate markings of applicable testing agency.

          - Flame-Spread Index:  [25] [75] [200] or less.

          - Smoke-Developed Index:  450 or less.

        - Regulatory Requirements:  Comply with applicable provisions in [the U.S. Architectural & Transportation Barriers Compliance Board's ADA-ABA Accessibility Guidelines] [and] [ICC/ANSI A117.1] for shower and dressing compartments designated as accessible.

      - PROJECT CONDITIONS

        - Field Measurements:  Verify actual locations of walls, columns, ceilings, and other construction contiguous with shower and dressing compartments by field measurements before fabrication.

Insert article for warranty requirements if required.  Availability of warranties varies among manufacturers and by type of compartment.  When special warranties are required, verify with Owner's counsel that special warranties are not less than remedies available to Owner under prevailing local laws.

- PRODUCTS

      - MATERIALS

Retain only those materials in this article that are referenced in other articles for specific compartments and accessories required.

        - Aluminum Castings:  ASTM B 26/B 26M.

        - Aluminum Extrusions:  ASTM B 221 (ASTM B 221M).

        - Brass Castings:  ASTM B 584.

        - Brass Extrusions:  ASTM B 455.

If heavier-than-minimum coating designations for hot-dip galvanized- or galvannealed-steel sheet are required, revise requirements in first paragraph below; verify availability with manufacturers.  Insert requirements if electrolytically zinc-coated steel sheet is required for any components.  See "Steel Compartments" Article in the Evaluations.

Retain first two paragraphs below for stainless-steel brackets (fittings) and for stainless-steel compartment panels.  Manufacturers make brackets (fittings) from either material and often do not state which is used.

        - Stainless-Steel Sheet:  ASTM A 666, Type 304, stretcher-leveled standard of flatness.

        - Stainless-Steel Castings:  ASTM A 743/A 743M.

Retain first two paragraphs below for plastic-laminate-faced dressing compartments.  Retain second option in "Particleboard" Paragraph below if required for LEED-NC, LEED-CI, or LEED-CS Credit IEQ 4.4.  Retain third option if required for LEED for Schools Credit IEQ 4.

Retain "Adhesives" Paragraph below if required for LEED for Schools Credit IEQ 4.

        - Adhesives:  Manufacturer's standard product that complies with the testing and product requirements of the California Department of Health Services' "Standard Practice for the Testing of Volatile Organic Emissions from Various Sources Using Small-Scale Environmental Chambers."

      - CMU BLOCK WITH CULTURED MARBLE, GRANITE, OR OTHER SOLID SURFACE PANEL COMPARTMENTS <Insert drawing designation> [In Development: suggested components for shower compartments; use with fiber cement board ceiling.  Pre-molded shower units may be used within restrooms only]

      - STAINLESS-STEEL COMPARTMENTS <Insert drawing designation>

Copy this article and re-edit for each product.

Insert number to complete drawing designation.  Use these designations on Drawings to identify each product.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of first two paragraphs and list of manufacturers below.  See Section 016000 "Product Requirements."

        - Manufacturers:  Subject to compliance with requirements, provide products by one of the following :

        - Basis-of-Design Product:  Subject to compliance with requirements, provide partitions by General Partitions Mfg. Corp.; www.generalpartitions.com or comparable product by one of the following:

          - Global Steel Products Corp; www.globalpartitions.com.

          - Substitutions:  See Section 01 25 00 - Substitution Procedures.

        - Configuration:     As shown on Drawings.

Insert another enclosure style, such as ceiling hung or floor anchored, in first paragraph below if required; verify availability and recommended use with manufacturers.  See Evaluations.

        - Enclosure Style:  Overhead braced

Some manufacturers weld face sheets together at corners; others do not.  Revise first paragraph below to suit Project.  Insert other requirements, such as panels and doors with no-sightline system; verify availability with manufacturers.  See Evaluations.

        - Panel and Pilaster Construction:  Seamless metal facing sheets, pressure laminated to core material; with continuous, interlocking molding strip or lapped-and-formed edge closures and with corners secured by welding or clips and exposed welds ground smooth.  Exposed surfaces shall be free of pitting, seam marks, roller marks, stains, discolorations, telegraphing of core material, or other imperfections.

Revise subparagraphs below to suit Project.  See Evaluations.

          - Core Material:  Manufacturer's standard, sound-deadening honeycomb of resin-impregnated kraft paper in thickness required to provide finished thickness of 1 inch (25 mm) for panels and 1-1/4 inches (32 mm) for pilasters.

          - Grab-Bar Reinforcement:  Provide concealed internal reinforcement for grab bars mounted on compartments.

If necessary, insert requirements for internal reinforcement if small compartment-mounted accessories such as soap holders are attached; see "Metal Compartments, General" Article in the Evaluations.

          - Tapping Reinforcement:  Provide concealed reinforcement for tapping (threading) at locations where machine screws are used for attaching items to compartments.

Retain first paragraph below if doors are required; delete if curtains are required.  Insert other requirements, such as panels and doors with no-sightline system; verify availability with manufacturers.  See Evaluations.

        - Door Construction:  Match panels; 1-inch (25-mm) finished thickness.

        - Facing Sheets and Closures:  Stainless-steel sheet of nominal thicknesses as follows:

Before revising subparagraphs below, see "Metal Compartments, General" Article in the Evaluations for a discussion of steel sheet thicknesses; a table correlates traditional gage numbers with specified decimal thicknesses.

Retain first option in first subparagraph below to maximize competition among listed manufacturers.  Second option eliminates some manufacturers or requires custom fabrications.

          - Panels:  Manufacturer's standard thickness, but not less than 0.031 inch (0.79 mm) [0.038 inch (0.95 mm)].

Retain subparagraph below if doors are required; delete if curtains are required.  Insert other requirements, such as panels and doors with no-sightline system; verify availability with manufacturers.  See Evaluations.

          - Doors:  Manufacturer's standard thickness, but not less than 0.031 inch (0.79 mm).

        - Pilaster Shoes:  Formed from stainless-steel sheet, not less than 0.031-inch (0.79-mm) nominal thickness and 3 inches (76 mm) high, finished to match hardware.

        - Brackets (Fittings):

Retain first subparagraph below for shower compartments; verify availability with manufacturers if inserting material other than aluminum.  Retain first, second, or third subparagraphs for dressing compartments; if retaining more than one, indicate locations of each type of fitting on Drawings or by inserts.  See Evaluations.

          - Dressing-Compartment Brackets:  Match toilet-compartment brackets specified in Section 10 21 13 "Toilet Compartments."

Textured finishes are available from some manufacturers, but textures vary.

        - Stainless-Steel Finish:    Match stainless-steel toilet-compartment finish, specified in Section 10 21 13 "Toilet Compartments," on exposed faces.  Protect exposed surfaces from damage by applying strippable, temporary protective covering before shipment.

      - PHENOLIC-CORE COMPARTMENTS <Insert drawing designation>

Copy this article and re-edit for each product.

Insert number to complete drawing designation.  Use these designations on Drawings to identify each product.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain one of first two paragraphs and list of manufacturers below.  See Section 016000 "Product Requirements."

        - Manufacturers:  Subject to compliance with requirements, provide products by one of the following :

        - Basis-of-Design Product:  Subject to compliance with requirements, provide partitions by General Partitions Mfg. Corp.; www.generalpartitions.com or comparable product by one of the following:

          - Global Steel Products Corp; www.globalpartitions.com.

          - Substitutions:  See Section 01 25 00 - Substitution Procedures.

        - Configuration:     As shown on Drawings.

Insert another enclosure style, such as ceiling hung or floor anchored, in first paragraph below if required; verify availability and recommended use with manufacturers.  See Evaluations.

        - Enclosure Style:  Overhead braced.

Insert other requirements, such as panels and doors with no-sightline system; verify availability with manufacturers.  See Evaluations.

        - Panel and Pilaster Construction:  Solid phenolic material consisting of solid phenolic-core panel with melamine facing on both sides fused to substrate during panel manufacture (not separately laminated) and with eased and polished edges.  Provide minimum 3/4-inch- (19-mm-) thick pilasters and minimum 1/2-inch- (13-mm-) thick panels.

Retain first paragraph below if doors are required; delete if curtains are required.  Insert other requirements, such as panels and doors with no-sightline system; verify availability with manufacturers.  See Evaluations.

        - Door Construction:  Match panels; 3/4-inch (19-mm) minimum thickness.

        - Pilaster Shoes:  Formed from stainless-steel sheet, not less than 0.031-inch (0.79-mm) nominal thickness and 3 inches (76 mm) high, finished to match hardware.

        - Brackets (Fittings):

Retain first subparagraph below for shower compartments; verify availability with manufacturers if inserting material other than aluminum.  Retain first, second, or third subparagraphs for dressing compartments; if retaining more than one, indicate locations of each type of fitting on Drawings or by inserts.  See Evaluations.

          - Dressing-Compartment Brackets:  Match toilet-compartment brackets specified in Section 10 21 13 "Toilet Compartments."

        - Phenolic-Core-Panel Finish:

          - Facing Sheet Finish:  [One color and pattern] [Two colors and patterns] in each room.

Solid "through-color" phenolic-core panels in last option in subparagraph below are available from Global Steel Products and perhaps from other manufacturers; verify availability.

          - Color and Pattern:    Match phenolic-core toilet compartments specified in Section 10 21 13 "Toilet Compartments" with manufacturer's standard dark-color core .

      - ACCESSORIES

Retain first paragraph below if doors are required; before revising, verify requirements of authorities having jurisdiction for accessible facilities.

        - Door Hardware and Accessories:  Manufacturer's standard design, heavy-duty, operating hardware and accessories.

Retain six subparagraphs below or revise to suit Project.

          - Material:   Stainless steel.

Hinge types vary in availability among manufacturers; most offer paired hinges in first subparagraph below.  Paired hinges can be either preset or adjustable; for preset hinges, insert requirements for hold-open angle.  Retain last option below only if retaining solid-polymer doors and pilasters configured to receive integral hinges.

          - Hinges:  Manufacturer's standard, continuous, cam type that swings to a closed or partially open position   .

Recessed latch-operating devices that are accessible to people with disabilities are available.  Verify requirements of authorities having jurisdiction.

          - Latch and Keeper:  Manufacturer's standard, recessed latch unit designed for emergency access and with combination rubber-faced door strike and keeper.  Provide units that comply with regulatory requirements for accessibility at compartments designated as accessible.

Retain option in first subparagraph below if in-swinging doors are required.

          - Clothing Hooks:  Manufacturer's standard clothing hooks in each dressing compartment; include one combination hook and rubber-tipped bumper at in-swinging doors, sized to prevent door from hitting wall panel or compartment-mounted accessories.

Retain first subparagraph below if out-swinging doors are required.

          - Door Bumper:  Manufacturer's standard, rubber-tipped bumper at out-swinging doors.

          - Door Pull:  Manufacturer's standard unit at out-swinging doors that complies with regulatory requirements for accessibility.  Provide units on both sides of doors at compartments designated as accessible.

Retain first paragraph below for shower compartments and overhead-braced dressing compartments.  Generally, clear-anodized finish is provided; revise if specific finish is required.

        - Overhead Bracing:  Manufacturer's standard, continuous, extruded-aluminum head rail or cap with antigrip profile; in manufacturer's standard finish.

Retain one of first two paragraphs below for openings into shower compartments and for curtained openings into dressing compartments; delete if only doors or shower dividers without curtains are required.  Generally, clear-anodized finish is provided; revise if specific finish is required.

        - Head Rail with Hooks:  Manufacturer's standard, continuous, extruded-aluminum head rail or cap with curtain hooks running in concealed track; with antigrip profile; in manufacturer's standard finish.

Retain first paragraph below if curtain is required.  Insert another fabric if required; coordinate washing temperature with fabric.

        - Curtain:  Flame-resistant, [polyester-reinforced vinyl fabric] [manufacturer's standard fabric] <Insert fabric> that is stain resistant, self-sanitizing, antistatic, and antimicrobial; launderable to a temperature of not less than [90 deg F (32 deg C)] <Insert temperature>.

Revise subparagraphs below to suit Project.

          - Flame Resistance:  Passes NFPA 701 tests when tested by a testing and inspecting agency acceptable to authorities having jurisdiction.

          - Labeling:  Identify fabrics with appropriate markings of applicable testing and inspecting agency.

          - Curtain Grommets:  Two-piece, rolled-edge, rustproof, nickel-plated brass; spaced not more than 6 inches (152 mm) o.c.; machined into top hem.

Revise first subparagraph below to suit Project.

          - Length:  Where curtain extends to a floor surface, size so that bottom hem clears finished floor by not more than 1 inch (25 mm) and not less than 1/2 inch (13 mm) above floor surface.  Where curtains extend to a shower-receptor curb, size so that bottom hem hangs above curb line and clears curb line by not more than 1/2 inch (13 mm).

          - Color and Pattern:   Match Architect's sample .

        - Soap Holder:  [Surface-mounted] [Recessed], seamless [stainless-steel] <Insert material> soap dish.

Seats in first paragraph below vary among manufacturers.  Verify availability before specifying.

        - Seats:  Manufacturer's standard, [panel-mounted] [wall-mounted] [or] [floor-mounted] benches.

          - Material:  [Wood] [Solid phenolic] [Molded plastic] [Stainless steel] <Insert material>.

          - Operation:  [Fixed] [Folding].

          - Finish:  [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] [Match enclosure panels] <Insert finish>.

Revise paragraph below to suit Project.

        - Anchorages and Fasteners:  Manufacturer's standard, exposed fasteners of stainless steel, chrome-plated steel, or solid brass, finished to match the items they are securing; with theft-resistant-type heads.  Provide sex-type bolts for through-bolt applications.  For concealed anchors, use stainless steel, hot-dip galvanized steel, or other rust-resistant, protective-coated steel.

      - FABRICATION

Retain one or both of first two paragraphs below to suit Project.

        - Overhead-Braced Compartments:  Provide manufacturer's standard, corrosion-resistant supports, leveling method, and anchors at pilasters and walls to suit floor and wall conditions.  Provide shoes at pilasters to conceal supports and leveling method.

Retain paragraph below if doors are required; revise to suit Project if door sizes and swings are not shown on Drawings; verify requirements of authorities having jurisdiction.

        - Door Sizes and Swings:  Unless otherwise indicated, provide 24-inch- (610-mm-) wide, in-swinging doors for standard shower and dressing compartments, and 36-inch- (914-mm-) wide, out-swinging doors with a minimum 32-inch- (813-mm-) wide, clear opening for compartments designated as accessible.

- EXECUTION

      - INSTALLATION

        - General:  Comply with manufacturer's written installation instructions.  Install compartments rigid, straight, level, and plumb.  Secure compartments in position with manufacturer's recommended anchoring devices.

          - Maximum Clearances for Dressing Compartment:

            - Pilasters and Panels:  1/2 inch (13 mm).

            - Panels and Walls:  1 inch (25 mm).

Retain subparagraph below for stirrup brackets.  For solid-polymer compartments, three brackets are recommended for panel-to-wall and panel-to-pilaster connections to prevent sagging of panel.

          - Stirrup Brackets:  Secure panels to walls and to pilasters with no fewer than [two brackets attached] [three brackets attached at midpoint and] near top and bottom of panel.

            - Locate wall brackets so holes for wall anchors occur in masonry or tile joints.

            - Align brackets at pilasters with brackets at walls.

Retain applicable mounting styles in first two paragraphs below; retain option if doors are required.

        - Overhead-Braced Compartments:  Secure pilasters to floor, and level, plumb, and tighten.  Set pilasters with anchors penetrating not less than 1-3/4 inches (44 mm) into structural floor unless otherwise indicated in manufacturer's written instructions.  Secure continuous head rail to each pilaster with no fewer than two fasteners.[  Hang doors to align tops of doors with tops of panels, and adjust so tops of doors are parallel with overhead brace when doors are in closed position.]

        - Floor-and-Ceiling-Anchored Compartments:  Secure pilasters to supporting construction, and level, plumb, and tighten.[  Hang doors and adjust so doors are level and aligned with panels when doors are in closed position.]

Retain first paragraph below if curtains are required.

        - Curtains:  Install curtains to specified length and verify that they hang vertically without stress points or diagonal folds.

Retain first paragraph below for prefabricated shower receptors.

        - Shower Receptors:  Install prefabricated shower receptors with drain gasket compression fit to outside diameter of waste pipe.

      - ADJUSTING

Retain this article if curtains or doors are required.  Revise hinge adjustment requirements to suit Project.  Verify accessibility requirements of authorities having jurisdiction.

        - Curtain Adjustment:  After hanging curtains, test and adjust each track or rod to produce unencumbered, smooth operation.  Steam and dress down curtains as required to produce crease- and wrinkle-free installation.  Remove and replace curtains that are stained or soiled or that have stress points or diagonal folds.

        - Hardware Adjustment:  Adjust and lubricate hardware according to hardware manufacturer's written instructions for proper operation.  Set hinges on in-swinging doors to hold doors open approximately 30 degrees from closed position when unlatched.  Set hinges on out-swinging doors to return doors to fully closed position.

END OF SECTION 10 21 16